DOCUMENTATION FOR VERSION 1.1 OF THE IBM BASIC TALKER PROGRAM

This  program is meant to be used on an IBM PC, XT, AT, or  their

clones.  It  will work with a monochrome, color or EGA  card.  In

color  it  uses a preset blue background  and  yellowish-greenish

characters with a menu in a red background and yellow characters.

In  monochrome the menu is in reverse video and the rest  of  the

screen is in high intensity with a black background.

This program allows the IBM PC to talk to the BASIC board at 9600

baud,  1  stop  bit, and no parity. These  parameters  cannot  be

changed (except maybe in a later version of this program).  While

in DOS and with the basic board turned off, type BASICBOA.  BASIC

will  come up with the ANAHEIM AUTOMATION sign on  message.  This

program  now  acts  like a terminal for talking  with  the  BASIC

board,  except  that  programs can be developed  also  with  text

editors  like WORDSTAR (non-document mode) and downloaded to  the

BASIC  board. Also programs developed, or altered, on  the  BASIC

board can be stored to disk. Finally, since each character is not

sent as it is typed but instead when the Enter key is pushed, the

user  may edit each line and then send it as an entire line.  The

user  can use the BASIC LIST command and list a range  of  lines,

then using the cursor keys move to the line desired, change  that

line, and send it with the Enter key.

The  cursor keys are fully functional, including Home,  and  End.

The  ten function keys are described partially in the menu.  They

are fully described below:

F1  - PRINT  - Toggles print mode on and off, so that  everything

printed  to  the  screen  is  also  printed  on  a

printer. Print mode cannot be toggled on if one is

not connected.

F2  - DELETE - Deletes  the line that the cursor is on and  pulls

everything  below it up, leaving the  bottom  line

blank.

F3  - INSERT - Pushes  the  line  that  the  cursor  is  on   and

everything  that is in the edit window  below  the

cursor down one line.

F4  - UP     - After  a  line  is edited and sent  to  the  BASIC

board,  the cursor goes to the bottom of the  edit

window.  This function key returns the  cursor  to

the next line after that line that was sent.

F5  - EXIT   - This exits back to DOS.

F6  - LOAD   - A  file  that was edited on a text editor  can  be

downloaded to the board by this means. BASIC  must

be  in RAM mode. The user is prompted for  a  file

name. LOAD implies load from disk.

F7  - STORE  - A  program on the BASIC board can be  uploaded  to

the  IBM  disk. The user is prompted  for  a  file

name. STORE implies store to disk.

F8  -  ^C    - This  sends a Control-C to the BASIC  board,  thus

stopping a program currently running.

F9  - TERM   - This  puts  the  BASIC  board  in  terminal   mode

allowing the user to talk directly to the SMC25 or

SMC20 as if the BASIC board did not exist.

F10 - RESET  - This  resets the BASIC board and sends a space  to

it.  The  transmit line is brought logic  low  (+9

volts  approximately) for about one  half  second.

The  line is then returned to logic high. This  is

known  as a break and an explanation can be  found

in  the  IBM BASIC manual  in  the  communications

section in the appendix.

Notes:

IBM PC is a trademark of International Business Machines Inc.

Wordstar is a trademark of Micropro Inc.

########################################################