SMC SMART INDEXER SOFTWARE

The SMC Smart Indexer Disk helps you create programs for your

Anaheim Automation Programmable Preset Indexers.  The disk covers

SMC Controller Chips from SMC25 through SMC38.

* Terminal Mode control of the Programmable Preset Indexer

* Stored program generation with no byte counting or calculations

* Multiple storted programs on disk

* Read programs from one indexer and download to another

* Anaheim Automation's SMC Editor allows individual comments

* Modify existing programs easily with the SMC Editor

* Function Key based SMC Editor

Function Key Definitions

F1  - INSERT        THIS PLACES COMMANDS ABOVE THE HIGHLIGHTED

LINE.

F2  - DELETE        THIS DELETED THE HIGHLIGHTED LINE.

F3  - ADD           THIS WILL PLACE COMMANDS AT THE END OF THE

PROGRAM.

F4  - CHANGE        THIS WILL REPLACE THE HIGHLIGHTED LINE WITH A

NEW COMMAND.

F5  - PRINT         THIS WILL SEND THE CURRENT PROGRAM TO THE

PARALLEL PORT OF YOUR COMPUTER.

F6  - SAVE          THIS WILL SAVE THE PROGRAM.  THE DIRECTORY OF

THE DISK YOU ARE RUNNING FROM WILL BE

DISPLAYED.  THE SOFTWARE WILL ASK YOU FOR A

FILENAME.

F7  - LOAD          THIS WILL LOAD A PROGRAM.  THE DIRECTORY OF

THE DISK YOU ARE RUNNING FROM WILL BE

DISPLAYED.  THE SOFTWEARE WILL ASK YOU FOR A

FILENAME.

F8  - TERM          THIS ALLOWS YOU TO TALK DIRECTLY TO THE

PROGRAMMABLY PRESET INDEXER AND TO CONTROL IT

WITH YOUR KEYBOARD.

F9  - QUIT          THIS WILL END THE "ADD" OR "INSERT" MODES.

F10 - EXIT          THIS WILL EXIT THE SOFTWARE.  BE SURE TO SAVE

ALL WORK BEFORE EXITING.

ALT F1 - COM        THIS WILL CHANGE THE SERIAL COMMUNICATION PORT

THAT YOU ARE USING.  MOST COMPUTERS CONTAIN

TWO COMMUNICATION PORTS: COM1 AND COM2.

ALT F2 -            THIS WILL INCRIMENT THE AXIS NUMBER

ALT F3 -            THIS WILL DECRIMENT THE AXIS NUMBER

ALT F4 -            THIS WILL CHANGE THE COLOR SCHEME OF THE

SOFTWARE.  FOR MONOCHROME MONITORS, MAKE SURE

YOU SEE A HIGHLIGHTED LINE IN THE PROGRAM.

ALT F5 -            THIS WILL CHANGE THE VERSION OF THE CHIP.  THE

CHOICES ARE: SMC25, SMC27, SMC28, SMC35,

SMC37, AND SMC38.  IF YOU HAVE A VERSION OF

THE CHIP THAT IS NOT LISTED, USE THE SMC35

SETTING.  IF THAT DOES NOT WORK, CALL THE

FACTORY FOR ASSISTANCE.

C (CLEAR)           THIS WILL CLEAR THE PROGRAM FROM THE SMC

EDITOR.

S (SEND)            THIS WILL SEND THE PROGRAM IN THE SMC EDITOR

TO THE PROGRAMMABLE PRESET INDEXER.  ANY

PROGRAM ALREADY RESIDENT IN THE PROGRAMMABLY

PRESET INDEXER WILL BE OVERWRITTEN.

V (VERIFY)          THIS WILL READ THE PROGRAM FROM THE

PROGRAMMABLE PRESET INDEXER, AND LOAD IT INTO

THE SMC EDITOR.

R (RESET)           THIS WILL SENT A RESET SIGNAL TO THE

PROGRAMMABLE PRESET INDEXER WHICH WILL RESET

THE UNIT.

G (GO)              THIS WILL RUN THE PROGRAM IN THE SMC EDITOR

A (AUTO ON)         THIS WILL SET THE AUTOSTART FLAG ON IN THE

PROGRAMMABLE PRESET INDEXER TO RUN THE PROGRAM

FROM LINE 1 WHEN POWER IS APPLIED TO THE UNIT.

O (AUTO OFF)        THIS WILL SET THE AUTORSTART FLAG OFF IN THE

PROGRAMMABLE PRESET INDEXER SO THAT THE UNIT

WILL NOT AUTOSTART WHEN POWER IS APPLIED.

ONLINE/OFFLINE

THE UPPER RIGHT HALF CORNER OF THE SCREEN WILL DISPLAY EITHER

"ONLINE" OR "OFFLINE".  IF THE SOFTWARE IS SET UP CORRECTLY, THE

SERIAL CABLE IS PROPERLY ATTACHED, AND THE PROGRAMMABLE PRESET

INDEXER HAS POWER, THE "ONLINE" MESSAGE SHOULD BE DISPLAYED.  IF

THE "OFFLINE" MESSAGE IS SHOWN, THEN RECHECK THE ALT FUNCTION KEYS

TO MAKE SURE THE CHIP NUMBER AND AXIS NUMBER IS SET UP CORRECTLY.

ALSO CHECK THE CABLE CONNECTOR AND THE BAUD RATE AND AXIS SETTING

ON THE INDEXER.

TERMINAL MODE

THIS MODE ALLOWS DIRECT COMMUNICATION BETWEEN YOUR COMPUTER AND THE

PROGRAMMABLE PRESET INDEXER.  YOU CAN MANUALLY CONTROL THE INDEXER

BY SENDING COMMAND FROM YOUR COMPUTER.  SEE THE PROGRAMMING MANUAL

FOR YOUR INDEXER FOR MORE INFORMATION.